American University of Beirut

Faculty of Engineering and Architecture

Spring 2013

Profs:

Riad Chedid

Shahwan Khoury

Joseph Costantine

Student Name:………………………………………

EECE 380- Engineering Electromagnetics

OPEN BOOK Exam:

- Programmable Calculators are not allowed

- Provide your answer on the computer’s card only

- ….

Problem 1:

Find the force exerted by Q1(3.0 m, 3.0 m, 3.0 m)=1 µC on Q2(6.0m, 9.0m, 3.0 m)=10 nC.

- F12= 2 N

- F12= 1.46 ax µN

- F12= 0.89 ax +1.78 ay µN

- F12=0.5 ax+ 2.2 az nN

Complete Solution

Problem 2:

Given a Volume defined by 1.0≤r≤3.0m, 0≤θ≤90º, 0≤Φ≤90º. Perform the necessary integration to find its volume.

- V= 12 m3

- V= 13.6 m3

- V=15.6 m3

- None of the above

Complete Solution

Problem 3:

A line charge with charge density 2.00 nC/m exists at y= -2.0 m, x=0. A charge Q=8 nC exists somewhere along the y-axis. Where must you locate Q so that the total electric field is zero at the origin?

- y= 2 m

- y= 4 m

- y=1.7 m

- y=0

Complete Solution

Problem 4:

An infinitely long line charge (ρL=21π nC/m) lies along the z-axis. An infinite area sheet charge (ρs=3 nC/m2) lies in the x-z plane at y=10 m. Find a point on the y-axis where the electric field intensity is zero.

- y=5 m

- y=2 m

- y=3 m

- y=7 m

Complete Solution

Problem 5:

Consider a circular disk in the x-y plane of radius 5 cm. Suppose the charge density is a function of radius such that ρs=12 ρ nC/cm2 (Where ρ is in cm). Find the electric field intensity at a point 20 cm above the origin on the z-axis.

- E=34 V/m az

- E=2 KV/m az

- E=3.4 V/ cm az

- E=6.7 KV/cm az

Complete Solution

Problem 6:

Given D=2ρ aρ + sinΦ az C/m2, find the electric flux passing through the surface defined by 2≤ρ≤4 m, 90º≤Φ≤180º, and z=4 m.

- Ψ=6C

- Ψ=2C

- Ψ=1.76 C

- Ψ=21 C

Complete Solution

Problem 7:

Determine the charge density at the point P(3 m, 4 m, 0) if the electric flux density is given as D=xyz az C/m2.

- ρv=6C/m3

- ρv=7.5C/m3

- ρv=12C/m3

- None of the above

Complete Solution

Problem 8:

For V=X+Y2Z (V), determine E.

- E=-ax-2yzay-y2az

- E=ax-2yay-y2az

- E=2yzay+4yaz

- E=-ax-2yzay

Complete Solution

Problem 9:

The potential field in a material with εr=10.2 is V=12xy2 (V). Find P-The electric polarization field.

- P= -ax-yay nC/m2

- P= -0.976y2ax-1.95xyay nC/m2

- P= 2yax+3xay nC/m2

- P= -12.6ax nC/m2

Complete Solution

Problem 10:

A plane defined by 3x+2y+z=6 separates two dielectrics. The first dielectric, on the side of the plane containing the origin has εr1=3 and E1=4 az V/m. The other dielectric has εr2=6. Find E2.

- E2= -1.2ax-2.3ay V/m

- E2= -0.76ax-0.2ay-0.34 az V/m

- E2= -0.428ax-0.285ay +3.858 az V/m

- E2= 0.34ax+2.3ay+5.4az V/m

Complete Solution

Problem 11:

Charge is distributed along the z-axis between z=± 5 m with a uniform density ρl= 20 nC/m. Determine E at (2,0,0)m

- 55 ax V/m

- 250 ax V/m

- 65 ax + 40 az V/m

- 167 ax V/m

- None of the above

Problem 12:

Three point charges each of 2 μC are located in air at the corners of an equilateral triangle that is 10 cm on each side. Find the magnitude of the force experienced by each charge.

- 0.624 N

- 0.866 N

- 1.8 N

- 1.23 N

- None of the above (the answer is 6.24)